Calibration system for the LDRD detector front-end electronics.

The vertex detector will be instrumented with a large number of the FPIX 2.1 custom integrated circuits developed at Fermilab. The FPIX chip design has the ability to receive an external pulse, which can be presented to a pre-selected number of input channels. We have designed an in situ calibration system to generate these pulses. The requirements for generating the pulse include:

- A negative going pulse is superimposed on a positive offset voltage.

- The amplitude and offset voltages can be set precisely, with good dynamic range.

- The pulse is generated at a precise time relative to a system clock.

- Programming of the pulse characteristics can be done via a computer or by a field programmable gate array.

We have developed a design and constructed a prototype module that satisfies these requirements. A block diagram of the design is shown in Figure 1. The hardware consists of small, low power, CMOS compatible integrated circuits. Programming and pulse triggering are done via four control lines. The pulse amplitude and offset are set with 10 bit precision.  Pulse timing is adjustable by varying a front panel delay control or by changing the timing of the external trigger.

Figure 1. Calibration system block diagram.

The prototype calibration board was developed on a 6U VME board. It has been successfully tested on the bench, where it was downloaded and triggered via a PC. Software to program the board was written in C.

We have connected the calibration system to the FPIX chip and the FPGA based readout system. The pulse characteristics were setup for an ~10 ns risetime and ~1.5 us fall time.

Programming the calibration board was done directly from the FPGA. The FPGA output FIFO was readout by 3 high speed parallel ports into a Linux based PC. Data were analyzed using ROOT. With the FPIX / sensor combination, we observe an average noise level of ~140 electrons, with no effort to shield the FPIX or to otherwise remove any environmental noise.