E1039 Proptube Preamp Card Test  Procedure

Pat McGaughey and Ming Liu

12/19/2019

Proptube preamp card test stand setup, normal operating parameters:

- Pulse generator AFG3102

  - 10kHz square wave

  - V_pp, >= 200mV

- Digital scope with differential probe TDS5034B

  - Chan-1, monitoring input test pulse (1 MOhm)

  - Chan-2, Regular probe, 1MOhm;

  - Chan-3, Differential probe to check output signal

- Proptube front panel, with stands

- Power supply, +/-5V, 1A each;

  - +5V,  I = 0.40 A

  - -5V,   I = 0.55 A

- Preamp card to be tested

  - 56   Ohm   version

  - 300 Ohm version

Normal output: V_pp = 1.0V  (ECL output)

=========================

Test logbook:

https://docs.google.com/spreadsheets/d/1jzI36KzsCZkU85pxJ0IMISJ86WhYzkeignXAFpBP5RA/edit?usp=sharing

=========================

Prop-Tube_PreApm-Card_Channel-Map-IMG_0078.jpg

Test bench @C129

Power Supply: +5V (0.4A) and -5V (0.55A)

Pulser: 10kHz square wave, V_pp > 200mV

Signals: with external pulser, output = Differential ECL signal V_pp = 1V

Preamp card output channel map:

From Right to Left: 1, 2,3, …, 16, 17, see photo below.