This report shows distribution of the signals over the banks of the US and PA3 devices.

The following signals going to the MGT directly are excluded:

- DATA_MGT[0:27]

- CLOCKDES[3:4]

- CLK[1:2]

After triplicating the DCLK_D and DCLK_DE, bank 64 did not provide enough 3,3V pins. The following 2 scenarios are worked out:

- DCLK_DE is not triplicated. Bank 64 has enough pins

- DCLK_DE is mapped on Bank 65 which is operated on 3,3V (2,5V). As this bank also contains 28 select-map signals, the configuration interface including JTAG also needs to operate on 3,3V (2,5V).  This will affect the schematic on multiple places.

Other scenarios (e.g. level-converters on the powerboard signals) could also be considered

It contains the following tables:

- All signals that go to the US banks.

- Signals grouped per US bank for scenario1

- Signals grouped per US bank for scenario2

- All signals that go to the PA3 banks.

- Signals grouped per PA3 bank for scenario1

- Signals grouped per PA3 bank for scenario2

| Signal | Block | Type | VCCI | #pins | subtotal | Bank |
| --- | --- | --- | --- | --- | --- | --- |
| GBTxRESETB | Reset | CMOS1V5 | 1V5 | 1 | 1 | 47 |
| RXRDY | GBTx | CMOS1V5 | 1V5 | 1 | 8 |  |
| TXRDY |  |  |  | 1 |  |  |
| TxDATAVALID |  |  |  | 1x3 |  |  |
| RxDATAVALID |  |  |  | 1x3 |  |  |
| CLOCKDES[1:2] |  | SLVS | 1V8 | 4 | 48 | 45/46/48 |
| DOUT[0:9] |  |  |  | 20 |  |  |
| DCLK[0:1] |  |  |  | 4 |  |  |
| DIN[0:9] |  | LVDS_18 |  | 20 |  |  |
| TXRDY_1 | GBTx1 | CMOS1V5 | 1V5 | 1 | 4 | 47 |
| TxDATAVALID1 |  |  |  | 1x3 |  |  |
| DCLK1[0:1] |  | SLVS | 1V8 | 4 | 24 | 45/46/48 |
| DIN1[0:9] |  | LVDS_18 |  | 20 |  |  |
| RXRDY_2 | GBTx2 | CMOS1V5 | 1V5 | 1 | 8 | 47 |
| TXRDY_2 |  |  |  | 1 |  |  |
| TxDATAVALID2 |  |  |  | 1x3 |  |  |
| RxDATAVALID2 |  |  |  | 1x3 |  |  |
| DOUT2[0:9] |  | SLVS | 1V8 | 20 | 44 | 45/46/48 |
| DCLK2[0:1] |  |  |  | 4 |  |  |
| DIN2[0:9] |  | LVDS_18 |  | 20 |  |  |
| GPIO[0:3] | SCA | CMOS1V5 | 1V5 | 4x3 | 22 | 47 |
| GPIO[4:11] |  |  |  | 8 |  |  |
| SCL, SDA |  |  |  | 2 |  |  |
| DATA_LVDS[0:27] | FireFly | LVDS_18 | 1V8 | 56 | 56 | 66/67 |
| DCTRL[0:4] |  | CMOS3V3 | 3V3 | 5x2 | 30 40 | 64 |
| DCLK[0:4]_D |  |  |  | 5x3 |  |  |
| DCLK[0:4]_DE |  |  | 3V3 3V3 | 5x1 5x3 |  | 64 65 |
| DIO[0:9] | PA3 | LVDS_18 | 1V8 | 20 | 32 | 66/67 |
| DIO[0:11] |  | CMOS1V8 |  | 12 |  |  |
| FX3DQ[0:31] | USB3 | CMOS1V8 | 1V8 | 32 | 50 | 68 |
| FX3CTL[0:12] |  |  |  | 13 |  |  |
| FX3RESET |  |  |  | 1 |  |  |
| FX3INT# |  |  |  | 1 |  |  |
| FX3PCLK |  |  |  | 1 |  |  |
| FX3UARTTX |  |  |  | 1 |  |  |
| FX3UARTrx |  |  |  | 1 |  |  |
| I2C | Power_Board | CMOS3V3 | 3V3 | 16 | 16 | 64 |
| Clock_Block | CLK[0] | LVDS | 1V8 | 2 | 2 | 66/67/68 |
| Total |  |  |  |  | 345 355 |  |

Scenario 1

SN65MLVD080 DE not triplicated

| Bank | #pins DIFF+se | Used | Voltage |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 44 | 48+3 | 0 | 0 | - |  |  |
| 45 | 48+3 | 116 | 1V8 | SLVS & LVDS GBTx (48) SLVS & LVDS GBTx1 (24) SLVS & LVDS GBTx2 (44) |  |  |
| 46 | 48+3 |  |  |  |  |  |
| 48 | 48+3 |  |  |  |  |  |
| 47 | 48+3 | 43 | 1V5 | Reset (1) GBTx (8) GBTx1 (4) GBTx2 (8) SCA(22) |  |  |
| 64 | 48+4 | 50 | 3V3 | FireFly CLK/CTRL (30) PowerBoard-I2C (16) CAN (4) |  |  |
| 65 | 48+4 | 28 | 1V5 | SelectMap (28) |  |  |
| 66 | 48+3 | 86 | 1V8 | FireFly DATA_LVDS (56) PA3 (30) |  |  |
| 67 | 48+3 |  |  |  |  |  |
| 68 | 48+3 | 50 |  | USB3 (50) |  |  |
| JTAG | 4 |  | 1V5 |  |  |  |
| dedicated | 6 |  |  | Select_map |  |  |

Scenario 2

SN65MLVD080 DE triplicated

| Bank | #pins DIFF+se | Used | Voltage |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 44 | 48+3 | 0 | 0 | - |  |  |
| 45 | 48+3 | 116 | 1V8 | SLVS & LVDS GBTx (48) SLVS & LVDS GBTx1 (24) SLVS & LVDS GBTx2 (44) |  |  |
| 46 | 48+3 |  |  |  |  |  |
| 48 | 48+3 |  |  |  |  |  |
| 47 | 48+3 | 43 | 1V5 | Reset (1) GBTx (8) GBTx1 (4) GBTx2 (8) SCA(22) |  |  |
| 64 | 48+4 | 45 | 3V3 | FireFly CLK/CTRL (25) PowerBoard-I2C (16) CAN (4) |  |  |
| 65 | 48+4 | 43 | 3V3 | DCLK[0:4]_DE (5x3) SelectMap (28) |  |  |
| 66 | 48+3 | 86 | 1V8 | FireFly DATA_LVDS (56) PA3 (30) |  |  |
| 67 | 48+3 |  |  |  |  |  |
| 68 | 48+3 | 50 |  | USB3 (50) |  |  |
| JTAG | 4 |  | 3V3 |  |  |  |
| dedicated | 6 |  |  | Select_map |  |  |

This needs the configuration part (JTAG + Bank65) to operate on 3,3V (or 2V5).

Following changes need to be implemented:

- Level converter (1,5->3,3V) in JTAG chain

- Level convertor for Program/Done/Init signals going to SCA.

| Signal | Block | Type | VCCI | #pins | Bank |
| --- | --- | --- | --- | --- | --- |
| GPIO[12:19] | SCA | CMOS1V5 | 1V5 | 8 | 1 |
| FLASH_DATA[0:7] | FLASH_ROM | CMOS3V3 | 3V3 | 8 | 4 |
| FLASH_CONTROL |  |  |  | 7 | 4 |
| CLOCK_CONTROL | Clock | CMOS2V5 | 2V5 | 6 | 6 |
| CLK3P, CLK3N |  | LVDS | 2V5 | 2 | 6 |
| CAN | CAN | CMOS | 3V3 | 4 | 4 |
| DIOP[0:9].DION[0:9] | FPGA_US | LVDS | 2,5V (1,8V) | 20 | 0 |
| DIO[0:11] |  | CMOS1V5 | 1V5 | 12 | 7 |
| D_SCRUB[0:31] |  | CMOS1V5 CMOS3V3 | 1V5 (VCONFIG) 3V3 (VCONFIG) | 32 | 5 |
| SCRUB_CONTROL |  |  |  | 6 | 5 |
| Clkdes | GBTx | LVDS | 2V5 | 8 | 6 |

Scenario1

| Bank | #pins | Used | Voltage |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 40 | 20 | 2V5 (1V8) | US-DIO (LVDS) |  |  |
| 1 | 32 | 8 | 1V5 | SCA |  |  |
| 2 | 32 |  |  |  |  |  |
| 3 | 32 |  |  |  |  |  |
| 4 | 28 | 19 | 3V3 | Flash Data/Control, CAN |  |  |
| 5 | 39 | 38 | 1V5 (Vconfig) | Scrubbing |  |  |
| 6 | 32 | 10 | 2V5 | Clocking, |  |  |
| 7 | 32 | 12 | 1V5 | US-DIO (SE) |  |  |
| JTAG | 5 | 5 | 1V5 (Vconfig) |  |  |  |

Scenario2

| Bank | #pins | Used | Voltage |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 40 | 20 | 2V5 (1V8) | US-DIO (LVDS) |  |  |
| 1 | 32 | 8 | 1V5 | SCA |  |  |
| 2 | 32 |  |  |  |  |  |
| 3 | 32 |  |  |  |  |  |
| 4 | 28 | 19 | 3V3 | Flash Data/Control, CAN |  |  |
| 5 | 39 | 38 | 3V3 (Vconfig) | Scrubbing |  |  |
| 6 | 32 | 10 | 2V5 | Clocking, |  |  |
| 7 | 32 | 12 | 1V5 | US-DIO (SE) |  |  |
| JTAG | 5 | 5 | 3V3 (Vconfig) |  |  |  |