sPHENIX MVTX Readout and Controls

Ming Liu et al

V1.0, July 2017

The proposed sPHENIX MVTX system (Monolithic-Active-Pixel-Sensor-based Vertec Detector) consists of ALICE ALPIDE chips, ALICE Readout Units (RU) and ATLAS FELIX PCIe boards, and the Power Supply distribution boards developed by LBNL for the ALICE ITS upgrade.

All hardware specs are already determined or to be fixed soon either by ALICE (ALPIDE and RU) and ATLAS (FELIX).

Here we summary the latest high level technical specifications of ALICE ALPIDE chip controls and operations, ALICE RU and ATLAS FELIX boards I/Os, controls and also the PS distribution and monitoring system.

REF:

pALPIDEfs datasheet, v1.0b, Mar 24, 2014

ALPIDE-3 Operation Manual – Draft (Feb. 2, 2016)

ALICE ITS Upgrade, Readout Electronics – WP10 (Jan. 06, 2016)

ALICE ITS Upgrade Report, WP10 Data Rate (May 1, 2015)

ALICE ITS Upgrade TDR (2014)

ATLAS FELIX v1.5  (2016)

sPHENIX CDR (2017, updated)

MVTX Readout and Controls – Overview

Figure 1 Over view of MVTX readout system.

MAPS operation on nutshell:

ALPIDE-3 design on nutshell

Block diagram and pinout. The serial data stream is 8b/20b encoded.

Figure 2 ALPIDE-3 Pad layout

Interface signals:

The main functional I/Os of ALPIDE-3 chip are listed here. The COMS I/Os are 1.8V compatible. The high-speed data line transmits serial data at the maximum rate of 24b x 40MHz = 960Mbps.

Typos:

V and I

The MCLK, DCTRL and DCLK differenctial ports are implemented with a custom designed differential transceiver cell.  This has been desgined with reference to the standard TIA/EIA-899 Electrical Characteristics of Multipoint-Low-Voltage-Differential-Signaling (M-LVDS).

Supply, ground and bias nets:

Recommended operating conditions:

Control interface and protocol

Chip identification: CHIPID[6:0]

HSB: CHIPID[6:4] are used to identify the chip operation mode – 1) Inner Barrel; 2) Master 3) Slave.  For MVTX application, all bits are set to 0,  b000xxxx .

CHIPID[3:0] are used to identify the location of a chip in a stave.

Control interfaces:

The slow control interfaces sere two purposes:

- provide read/write access to internal registers, commands, configuration and memories;

- distribute trigger commands and broadcast synchronous signals

For the MVTX application (ALICE ITS/IB mode), only the differential DCNTRL port is used.

Figure 3 Illustration of chip identification and geographic address allocation.

The nie(9) chips on the IB module are directly connected to a shared control differential line using the DCNTRL port. The control bus is entirely based on differential singaling and has multipoint topology.

Control transactions format:

Figure 4 Format of a single charachter ecchanged on the control bus.

Figure 5 Format  of valid transactions on the control bus. Note for MVTX, all chips are operating in the Master Mode.

The ALPIDE-3 control interface provideds access to the chip internal registers and data path memories, with 16-bit wide address field and 16-bit wide data payload.

Figure 6  Valid opcodes of control transactions.

Here are a few important registers:

Periphery Control Registers:

Pixel CFG Registers:

In each pixel there are two wriable registers: 1) te Mask register and 2) the Pulse Enable register.

Pixel CFG Register 1:

Pixel CFG Register 2:

DACs setting registers:

For each of DACs, there are 8-bit setting field.  The default values are show below.

Summary of ALPIDE-3 registers and memory addressing.

Triggering and Timing

There are two readout modes for ALIPIDE chips – 1) triggered and 2) Continuous.

For sPHENIX we foresee to use the trigger mode as sPHENIX data taking will be mainly based triggers (maximum rate = 15kHz).

A readout STROBE signal will be generated after receiving an external trigger.  The STROBE length is programmable, the default value is 50ns (less than one RHIC clock).

The expected sPHENIX LVL-1 trigger latency is about 4 us, based on our experience from PHENIX operation at RHIC.

Figure 7 Trigger and Strobe decay

RHIC clock is 9.4MHz, much slower than the LHC clock of 40MHz. All sPHNIX and MVTX trigger and timing is synchronized to the 9.4MHz RHIC clock.

We plan to operate ALIPDE chips at the same 40MHz LHC clock. This new 40MHz clock will be generated and phase locked to the RHIC 9.4MHz clock, mostly likely at FELIX end.

A dedicated daughter card will be developed by the BNL ATLAS group for the next version FELIX board v.20, to allow 9.4MHz RHIC clock as input.

Data Readout

We will use the high-speed data transmission line to send data from ALPIDE chip to the RU through FireFly cables.  The serial port implements a clock synchronous transmission based on the 600MHz clock produced by the internal PLL. Data are 8b/10b encoded with the LSB launched first. Further more, a bit is sent on every clock edge. It is important to note that data area loaded for transmission 30-bit at a time – 3 characters of 10-bit encoded bytes.  For MVTX, the 600MHz clock signal is directly used to produce DDR stream at 1.2Gbps.

Data Format:

We will use the same data format defined by the ALICE ITS upgrade. The valid data words are identified by predefined prefix bit strings.

Data format rules:

The Front-End Electronics - Readout Unit (RU)

RU is developed by the ALICE experiment to readout out ALICE/ITS detectors. We will use RU with minimal modification for the sPHENIX MVTX system, one RU per stave of 9 ALIPIDE chips. The RU interfaces the sensors on the stave, control and trigger systems of the sPHENIX experiment. RU collects the high-speed data stream from the sensors and convert them into optical signal through the GBT optic links to the beck-end electronics (currently envisioned to use FELIX boards developed by ATLAS) in the Counting House. It manages control lines which distribute RHIC clock, sPHENIX trigger and other slow control commands. RU also implements ancillary functions such as power supply control and monitoring.

The main functionalities of the Readout Unit are summarized here:

- Receive and distribute clock and trigger to the sensors

- Collect high-speed data stream from sensors and send repackaged data to the Beck-End (FELIX) via TGB links,

- Handle busy signal from the sensors

- Monitoring the sensor status

- Perform power monitoring and safety control & interlock

A single RU  serves one full stave composite of 9 ALPIDE sensors, hosting 3 GBT links paired with 2 FPGA, each FPGA can read up to 16 links from the stave. For MVTX application, only one FPGA is used for readout, leaving the 2nd one as a spare for backup. There are a total of 48 RUs in the MVTX readout system.

Figure 8 Diagram of MVTX Readout Unit. One RU serves one 9-chip stave.  The Beck-End are served by FELIX boards hosted on sPHENIX DAQ Servers in the counting house.

The GBT Versatile Link payload bandwidth is 3.2Gb/s.  Here we summarize the RUs and GBTx chipsets needed to readout the entire 48 MVTX detector staves for the maximum design rates.

Figure 9  Maximum data rate for each MVTX layer

Here we show the MVTX data and control routing schematic layout. It is worth to mention that for the sPHENIX baseline program (200kHz Au+Au), a single GBTx chip will provide enough bandwidth and the others could be used to hot-swap fiber links in case of connection problems. A MVTX stave is connected to the first connector A, with clock and control (2 lines) and 9 data lines running over the 12 available FireFly copper links.

sPHENIX MVTX actual bandwidth requirements:

The projected maximum collision rates for sPHENIX are summarized below:

ALICE has simulated data rate based on Pb+Pb collisions at the top collision rate of 200kHz (ALICE projected rate for Run-3 is 50kHz). With 2us shaping time of ALPIDE chip (the fastest shaping mode, also planned for sPHENIX MVTX operation), the estimated RU average data rate at 200kHz triggered readout is [Gb/s]:

- Layer-0:  4.8 Gb/s, or 1/2 of the hardware bandwidth limit (9.6Gb/s)

- Layer-1:  3.2 Gb/s, or 1/3 of the hardware bandwidth

- Layer-2:   2.4 Gb/s, or ¼  of the hardware bandwidth

Figure 10 ALICE RU average data rates for Pb-Pb collisions at 200kHz, with 2us ALPIDE shaping time. Both triggered and continuous readout modes are simulated. The MVTX/sPHENIX is designed to operate in the triggered mode with maximum rate of 15kHz.

The average event multiplicity of Au+Au collisions in sPHENIX is estimated about 1/5 – 1/4 of the multiplicity of Pb+Pb collisions at ALICE/LHC.

Figure 11 Event track multiplicities of p+p collisions from sPHENIX  and ALICE.

Furthermore, sPHENIX is designed to take data at 15kHz trigger rate. So the expected data rate from sPHENIX is about ¼ x 15/200 of ALICE/ITS IB (~1.9%).

The expected MVTX average RU data rates for Au+Au collisions at RHIC are:

- Layer 0:  0.1 Gb/s

- Layer 1:  0.07 Gb/s

- Layer 2:  0.05 Gb/s

They are well below the hardware limit of 9.6Gb/s per RU with 3 GBT links.  MVTX has the capacity to take all 200kHz Au+Au collisions at RHIC! For the p+p collisions at RHIC, since the event multiplicity is very low compared to Au+Au collisions, the expected data rate is order of magnitude below the Au+Au rate.  Later, one can consider to develop MVTX-based trigger for sPHENIX.

The overall data rate from MVTX is:

0.1 x 12 + 0.07 x 16 + 0.05 x 20 = 3.3Gb/s

The sPHENIX DAQ bandwidth is designed to handle 80Gb/s data to HPSS.

The Back-End Electronics – FELIX

FELIX board developed by BNL for the ATLAS upgrade project is chosen as the default path for MVTX/sPHENIX beck-end option.

FELIX has 48 bi-directional optical links that can be coupled to RU GBT links.

One FELIX board will readout 8 RUs, using 24 fiber links. A total of 6 FELIX boards are required to readout all 48 RUs, with one FELIX board per Server that does event building and data compression.

Basic functions of FELIX:

- PCIe Gen3 x 16 lane

- 48 bi-directional optical links upto 14 Gb/s

- With circuit to interface Timing & Trigger Control system (TTC) for both LHC and RHIC experiments

- Micro-controler to support FPGA reprogramming and firmware update

Figure 12 Layout of the MVTX readout and sPHENIX DAQ  system.

The current FELIX version 1.5 can only operate with LHC clock of 40MHz. A new version 2.0 is designed to be able to handle 10MHz RHIC clock through an on board Timing & Trigger Mezzanine card. We foresee this new FELIX boards will be used for both MVTX and TPC detectors in the PHENIX experiment.

Figure 13 ATLAS FELIX board will be used for the sPHENIX MVTX readout.

Power System

The MVTX power system is based on ALICE/ITS-IB system developed by LBNL.

The system distribute clean analogy, digital and bias posers to the sensors, and also provide detector health status through remote readout.  A near final prototype will be available for R&D at LANL by the end of this summer, 2017.